A complex column having one sidestream W1 is operating at total reflux. The feed composition, relative volatilities, and other specifications for this column are as follow

| Component | Xi | αi | Other specifications |
| --- | --- | --- | --- |
| 1 | 1/3 | 1 | N=5, p=3, total condenser, F=100 mol/h, and total reflux operation |
| 2 | 1/3 | 2 |  |
| 3 | 1/3 | 3 |  |

- Compute the values of bi/di, w1i/di, D, and W1 when it is specified that bb/db=16 and w1b/db=10

- On the basis of the following sets of bi/di's and w1i/di's at D=23.0808 and W1=46.9078, compute the bi/di's and w1i/di's at D=34.6522 and W1=38.2494 by use of the θ method of convergence for complex columns.

| Component | Xi | αi | (bi/di)1 | (w1i/di)1 | Other specifications |
| --- | --- | --- | --- | --- | --- |
| 1 | 1/3 | 1 | 16 | 10 | N=5, p=3, total condenser, F=100 mol/h, and total reflux operation |
| 2 | 1/3 | 2 | 1 | 2.5000 |  |
| 3 | 1/3 | 3 | 0.1975 | 1.1111 |  |